PROJECT 2: Use of buzzer, Distance sensor or LDR sensor and leds

Once uploaded, it will turn green and stay green until you reach the distance threshold that is applied. Once it reaches it, the green led will close and the red led will turn on and the buzzer will go off in 1 second intervals. Additionally, this can be used with a LDR / light sensor instead of a distance sensor (anything added as notes can be used and to replace the existing lines to change it.

Code:

const int gled = 13;

const int rled = 2;

const int buzz = 11;

const int trig = 4; // You can add int ldr = A0 for the alternative

const int echo = 3; //

void setup() {

// put your setup code here, to run once:

pinMode (gled, OUTPUT);

pinMode (rled, OUTPUT);

pinMode (trig, OUTPUT); // pinMode (ldr, INPUT) ;

pinMode (echo, INPUT); //

pinMode (buzz, OUTPUT);

Serial.begin(9600);

}

void loop() {

// put your main code here, to run repeatedly:

long duration, distance; //

digitalWrite(trig, LOW); // int value = analogRead(ldr);

delayMicroseconds(2); // Serial.println(value);

digitalWrite(trig, HIGH); //

delayMicroseconds(10); //

digitalWrite(trig, LOW); //

duration = pulseIn(echo, HIGH); // value = analogRead(sensor);

distance = (duration*.034)/2; //

Serial.print("distance: "); // Serial.println(value);

Serial.println(distance); //

delay(100); //

if (distance < 30) { // if (value>=300){

digitalWrite(gled, LOW);

digitalWrite(rled, HIGH);

tone(buzz, 1000);

delay(1000);

noTone(buzz);

delay(1000);

}else{

digitalWrite(gled, HIGH);

digitalWrite(rled, LOW);

noTone(buzz);

}

}